NAND2TETRIS   ASSIGNMENT N

Name : Sukhvinder Kaur

[A] HalfAdder

Schematic diagram :

Boolean Expression & Truth Table :

Code :

CHIP HalfAdder {

IN a, b;

OUT sum,

carry;

PARTS:

Xor(a=a, b=b, out=sum );

And(a=a, b=b, out=carry );

}

[B] FullAdder

Schematic Diagram :

Boolean Expression & Truth Table :

Code :

CHIP FullAdder {

IN a, b, c;  // 1-bit inputs

OUT sum,     // Right bit of a + b + c

carry;   // Left bit of a + b + c

PARTS:

Xor(a=a, b=b, out=ab);

Xor(a=ab, b=c, out=sum);

And(a=a, b=b, out=c1);

And(a=a, b=c, out=c2);

And(a=b, b=c, out=c3);

Or(a=c1, b=c2, out=c12);

Or(a=c12, b=c3, out=carry);

}