ASSIGNMENT-2

[A] Implement XOR gate using NAND :-

Boolean Expression :

Y’ = (AB’+A’B)’

Y = Y’’= ((AB’)’ .( A’B)’)’

Code :

CHIP Xor {

IN a, b;

OUT out;

PARTS:

Nand(a=a, b=a, out=x);

Nand(a=b, b=b, out=y);

Nand(a=a, b=y, out=z);

Nand(a=b, b=x, out=z1);

Nand(a=z, b=z1, out=out);

}

Truth Table :

| a | b | Out |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

[B] Implement Multiplexer (2-Way MUX) using Nand :-

Code:

CHIP Mux16 {

IN a[2], b[2], sel;

OUT out[2];

PARTS:

Not(in=sel , out=nsel);

And(a=a[0], b=nsel, out=x);

And(a=a[1], b=nsel, out=x1);

And(a=b[0], b=sel, out=y);

And(a=b[1], b=sel , out=y1);

Or(a=x, b=y , out=out[0]);

Or(a=x1 , b=y1 , out=out[1]);

}

[C] Implement Multiplexer (4-Way MUX) using Nand :-

Code:

CHIP Mux16 {

IN a[4], b[4], sel;

OUT out[4];

PARTS:

Not(in=sel , out=nsel);

And(a=a[0], b=nsel, out=x);

And(a=a[1], b=nsel, out=x1);

And(a=b[0], b=sel, out=y);

And(a=b[1], b=sel , out=y1);

Or(a=x, b=y , out=out[0]);

Or(a=x1 , b=y1 , out=out[1]);

}

[D] Implement DMux using Nand :-

CODE:

CHIP DMux {

IN in, sel;

OUT a, b;

PARTS:

Nand(a=sel, b=sel, out=notSel)

Nand(a=in, b=notSel, out=x);

Nand(a=x, b=x, out=a);

Nand(a=in, b=sel, out=y);

Nand(a=y, b=y, out=b);

}