ASSIGNMENT-1

AIM:  Implement NOT, AND OR gates using NAND gates using NAND2TETRIS Simulator SCHEMATIC DIAGRAM, BOOLEAN EXPRESSION AND TRUTH TABLE

Name: Sukhvinder Kaur

Section: D

Roll NO: 2501010272

CODE:

CHIP Not {

IN in;

OUT out;

PARTS:

Nand(a=in, b=in, out=out);

}

Code:

CHIP And {

IN a, b;

OUT out;

PARTS:

Nand(a=a, b=b, out=temp);

Nand(a=temp, b=temp, out=out);

}

Code:

CHIP Or {

IN a, b;

OUT out;

PARTS:

Nand(a=a, b=a, out=nota);

Nand(a=b, b=b, out=notb);

Nand(a=nota, b=notb, out=out);

}